# MATERIAL SAFETY DATA SHEET

MANUFACTURER:  	AMERICAN COATINGS		EMERGENCY TELEPHONE NO.

P.O. BOX 1426			1-800-424-9300 (CHEMTREC)

TOMBALL, TX  77377-1426

281-351-1776

FAX 281-351-6348

REVISION DATE:	November 10, 2011		NOTE:	16 SECTIONS

## SECTION 1 – PRODUCT IDENTIFICATION

PRODUCT NUMBER:	TH760

PRODUCT NAME: 	EPOXY THINNER

PRODUCT CLASS:        SOLVENT/THINNER

## SECTION 2 – HAZARDOUS INGREDIENTS

OCCUPATIONAL	      VAPOR

INGREDIENT		           	PERCENT  	EXPOSURE LIMITS               PRESSURE

(CAS #)				          TLV-TWA       OSHA-PEL          mm/Hg

78-93-3

*METHYL ETHYL KETONE	  40 %              200 ppm            200 ppm               71.2 @ 20 C

1330-20-7

*XYLENE			 20 %               100 ppm             100 ppm              9.5 @ 20 C

71-36-3

*BUTANOL                                    40 %                50 ppm               50 ppm               4.4 @ 20 C

* ARE LISTED ABOVE IS SUBJECT TO THE REPORTING REQUIREMENTS OF

SECTION 313

****  NONE LISTED AS A CARCINOGEN IN NTP, IARC OR BY OSHA  ****

TH760

SECTION 3 – HAZARDS IDENTIFICATION

EYES: IRRITATING to the eye.

SKIN CONTACT: MAY CAUSE SEVERE IRRITATION.  MAY CAUSE SENSITIZATION OR ALLERGIC REACTION.  CAN BE ABSORBED THROUGH THE SKIN.

INHALATION:  BREATHING HIGH VAPOR CONCENTRATIONS OR PROLONGED BREATHING Of LOWER CONCENTRATIONS CAN CAUSE NOSE AND THROAT IRRITATION AND MAY CAUSE HEADACHE, DIZZINESS, AND LOSS OF CONSCIOUSNESS.

INGESTION:  LIQUID IS MODERATELY TOXIC AND MAY BE Harmful if swallowed.  MAY PRODUCE CNS DEPRESSION.

PRIMARY ROUTE (S) OF ENTRY: SKIN CONTACT, INHALATION, INGESTION, AND EYE CONTACT

SECTION 4 – FIRST AID MEASURES

FIRST AID PROCEDURES:

EYE CONTACT: FLUSH IMMEDIATELY WITH LARGE AMOUNTS OF WATER FOR AT LEAST 15 MINUTES.  TAKE IMMEDIATLEY TO A PHYSICIAN FOR MEDICAL TREATMENT.

SKIN CONTACT: WASH AFFECTED AREAS WITH SOAP AND WATER.  REMOVE CONTAMINATED CLOTHING.  CONSULT PHYSICIAN IF IRRITATION PERSISTS.

INHALATION: REMOVE TO FRESH AIR.  RESTORE BREATHING, TREAT SYMPTOMATICALLY.  CONSULT A PHYSICIAN.

INGESTION: DO NOT INDUCE VOMITING.  IF VOMITING OCCURS SPONTANEOUSLY KEEP HEAD BELOW HIPS TO PREVENT ASPIRATION OF LIQUID INTO THE LUNGS.  GET MEDICAL ATTENTION.

CONTACT A PHYSICIAN OR POISON CONTROL IMMEDIATELY.  NEVER GIVE ANYTHING BY MOUNTH TO AN UNCONSCIOUS PERSON.

TH760

SECTION 5 – FIRE FIGHTING MEASURES

FLAMMABILITY CLASSIFICATION:

OSHA:	 FP < 73 F & BP =>100 F		LEL:	1.1

FLAMMABLE LIQUID CLASS IB

FLASH POINT (TCC)   45 F

EXTINGUISHING MEDIA:

FOAM:   X  	ALCOHOL FOAM:   X        CO2:   X            WATER FOG:   X         OTHER:     _

FIRE & EXPLOSION HAZARD DATA: KEEP CONTAINERS TIGHTLY CLOSED.

ISOLATE FROM HEAT, SPARKS, ELECTRICAL EQUIPMENT, AND OPEN FLAME.  CLOSED

CONTAINERS MAY EXPLODE WHEN EXPOSED TO EXTREME HEAT.

FIRE FIGHTING PROCEDURES: WARNING FLAMMABLE.  CLEAR FIRE AREA OF

UNPROTECTED PERSONNEL.  DO NOT ENTER CONFINED FIRE SPACE WITHOUT FULL

BUNKER GEAR (HELMET WITH FACE SHIELD, BUNKER COATS, GLOVES AND RUBBER

BOOTS)  INCLUDING A POSITIVE PRESSURE NIOSH APPROVED SELF-CONTAINED

BREATHING APPARATUS.  COOL FIRE EXPOSED CONTAINERS WITH WATER.

SECTION 6 – ACCIDENTAL RELEASE MEASURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED:  FLAMMABLE!

ELIMINATE ALL IGNITION SOURCES.  WEAR APPROPRIATE RESPIRATOR AND PROTECTIVE CLOTHING.

WASTE DISPOSAL METHOD:  IN CASE OF SPILLAGE, ABSORB WITH INERT MATERIAL AND DISPOSE OF IN ACCORDANCE WITH APPLICABLE REGULATION

SECTION 7 –  HANDLING AND STORAGE

Keep container closed, upright when not in use.  Store in cool, dry, well-ventilated area.  Avoid prolonged storage temperatures above 100o F.  Use caution when pouring.  Do not weld or flame cut on empty container.

TH760

SECTION 8 – EXPOSURE CONTROL/PERSONAL PROTECTION

RESPIRATORY PROTECTION: WEAR APPROPRIATE PROPERLY FITTED RESPIRATOR (NIOSH/OSHA APPROVED) DURING AND AFTER APPLICATION.  MECHANICAL VENTILATION IS RECOMMENDED IF IN A CONFINED OR RESTRICTED AREA.

VENTILATION:  USE WITH ADEQUATE VENTILATION.  DO NOT BREATHE VAPORS OR SPRAY MIST.

PROTECTIVE GLOVES: USE OIL RESISTANT GLOVES IF CONTACT IS LIKELY

EYE PROTECTION: USE CHEMICAL SAFETY GOGGLES

OTHER PROTECTIVE EQUIPMENT: OIL RESISTANT APRON OR COVERALS, EMERGENCY EYE WASH AND SHOWER STATIONS.

HYGIENIC PRACTICES: WASH THOROUGHLY AFTER HANDLING.  NON-STATIC CREATING CLOTHING AND CONDUCTIVE SHOES SHOULD BE WORN.

### SECTION 9 – PHYSICAL AND CHEMICAL DATA

BOILING RANGE:	175 F – 285° F               		WT./GALLON    6.85 #/GAL  0.2

EVAPORATION RATE:

FASTER:           		SLOWER:     xx    	THAN ETHER

% VOLATILE VOLUME: 100 %

VAPOR DENSITY:

HEAVIER:     xx    	LIGHTER:           	THAN AIR

MATERIAL VOC:  6.85 LB/GAL

TH760

SECTION 10 – STABILITY AND REACTIVITY DATA

STABILITY:				STABLE    XX			UNSTABLE ____

HAZARDOUS POLYMERIZATION: 	MAY OCCUR ___		WILL NOT OCCUR  XX

HAZARDOUS DECOMPOSITION PRODUCTS: CARBON MONOXIDE, CARBON DIOXIDE

CONDITIONS TO AVOID: HEAT, SPARKS, FLAME AND CONTACT WITH STRONG OXIDIZING AGENTS.

INCOMPATIBILITY (MATERIALS TO AVOID): OXIDIZING AGENTS

SECTION 11 – TOXICOLOGICAL PROPERTIES

SEE SECTION 2

SECTION 12 – ECOLOGICAL INFORMATION

NO INFORMATION available

SECTION 13 – DISPOSAL CONSIDERATIONS

EPA Waste No.: None

DISPOSAL METHOD: Place in separate, appropriate, closed container in accordance with all applicable local, State, and Federal regulations.  This material has not been tested by Toxicity Characteristic Leaching Procedure (TCLP.)

TH760

SECTION 14 – TRANSPORTATION INFORMATION

DOT PROPER SHIPPING NAME:  PAINT RELATED MATERIAL

DOT HAZARD CLASS:  3				HAZARD SUBCLASS:   NA

DOT UN/N NUMBER: UN1263				PACKING GROUP: II

SECTION 15 – REGULATORY INFORMATION

U.S. FEDERAL REGULATIONS:

OSHA:  Hazardous by definition of Hazard Communication Standard (29 CFR 1910.1200)

INTERNATIONAL REGULATIONS:

CANADIAN WHMIS: THIS MSDS has been prepared in compliance with Controlled Product Regulations except that 16 sections were used rather than the 9 sections listed.  Canadian WHMIS Class: NO information AVAILABLE.

SECTION 16 – OTHER INFORMATION

HMIS:	health: 2	Flammability: 3	reaCtivity: 0

THE INFORMATION CONTAINED HEREIN IS BASED ON THE DATA AVAILABLE TO US AND IS BELIEVED TO BE CORRECT.  AMERICAN COATINGS MAKES NO WARRANTY, EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THESE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.  AMERICAN COATINGS ASSUMES NO RESPONSIBILITY FOR INJURY FROM THE USE OF THE PRODUCT DESCRIBED HEREIN.