# MATERIAL SAFETY DATA SHEET

MANUFACTURER:  	AMERICAN COATINGS		EMERGENCY TELEPHONE NO.

P.O. BOX 1426			1-800-424-9300 (CHEMTREC)

TOMBALL, TX  77377-1426

281-351-1776

FAX 281-351-6348

dATE:			9/14/07

NOTE:			16 SECTIONS

SECTION 1 – PRODUCT IDENTIFICATION

- PRODUCT NUMBER:	TC7B

- PRODUCT NAME: 	BLUE CAULK PART B  (100% SOLIDS)

PRODUCT CLASS:        EPOXY COATING

SECTION 2 – HAZARDOUS INGREDIENTS

OCCUPATIONAL	      VAPOR

INGREDIENT		           	PERCENT  	EXPOSURE LIMITS               PRESSURE

(CAS #)				          TLV-TWA       OSHA-PEL          mmHg

68953-36-6

POLYAMIDOAMINE/TOFA/

TEPA/AEPA			     71 %            N/E              N/E      	       N/A

112-57-2

TETRA ETHYLENE PENTAMINE    2 %              N/E              N/E         	        N/A

TRADE SECRET

MODIFIED POLY ETHYLENE

POLYAMINE ADDUCTS                < 2 %              N/E               N/E                            < 0.1 @ 20 C

112945-52-5

SILICA, FUMED, AMORPHOUS	  < 5 %              2 mg/m3        20 mppcF	         N/A

14807-96-6

TALC				    18 %	           2 mg/m3	    2 mg/m3	         N/A

* ARE LISTED ABOVE IS SUBJECT TO THE REPORTING REQUIREMENTS OF

SECTION 313

**** NONE ARE LISTED AS A CARCINOGEN IN NTP, IARC OR BY OSHA ****

TC7B

SECTION 3 – HAZARDS IDENTIFICATION

ACUTE EXPOSURE:

SKIN:  cORROSIVE, MAY CAUSE SKIN BURING.  hARMFUL IF ABSORBEDTHROUGH SKIN.

INHALATION:  HARMFUL IF INHALED.

EYES:  CORRISIVE.  DIRECT CONTACT WITH EYES WILL CAUSE SEVERE BURNS AND MAY

CAUSE PERMANENT DAMAGE.

INGESTION:  Harmful if swallowed.

PRIMARY ROUTE (S) OF ENTRY: SKIN CONTACT, INHALATION, INGESTION, AND EYE

CONTACT

SECTION 4 – FIRST AID MEASURES

FIRST AID PROCEDURES:

EYE CONTACT: FLUSH IMMEDIATELY WITH LARGE AMOUNTS OF WATER FOR AT LEAST 15 MINUTES.  IMMEDATELY TAKE TO A PHYSICIAN FOR MEDICAL TREATMENT.

SKIN CONTACT: WASH AFFECTED AREAS WITH SOAP AND WATER.  REMOVE CONTAMINATED CLOTHING.  CONSULT PHYSICIAN IF IRRITATION PERSISTS.

INHALATION: REMOVE TO FRESH AIR.  RESTORE BREATHING, TREAT SYMPTOMATICALLY.  CONSULT A PHYSICIAN.

INGESTION: DO NOT INDUCE VOMITING.  IF VOMITING OCCURS SPONTANEOUSLY KEEP HEAD BELOW HIPS TO PREVENT ASPIRATION OF LIQUID INTO THE LUNGS.  CONSULT PHYSICIAN OR POISON CONTROL IMMEDIATELY.  NEVER GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS PERSON.

TC7B

SECTION 5 – FIRE FIGHTING MEASURES

FLAMMABILITY CLASSIFICATION:

- OSHA:	 FP => 200 F     				LEL:	N/E

- COMBUSTIBLE LIQUID CLASS IIIB

FLASH POINT (TCC)   > 200 F

- EXTINGUISHING MEDIA:

FOAM:           	ALCOHOL FOAM:               CO2:                   WATER FOG:                OTHER:   X _

FIRE & EXPLOSION HAZARD DATA: KEEP CONTAINERS TIGHTLY CLOSED.

ISOLATE FROM HEAT, SPARKS, ELECTRICAL EQUIPMENT, AND OPEN FLAME.  CLOSED

CONTAINERS MAY EXPLODE WHEN EXPOSED TO EXTREME HEAT.

FIRE FIGHTING PROCEDURES: WARNING COMBUSTIBLE.  CLEAR FIRE AREA OF

UNPROTECTED PERSONNEL.  DO NOT ENTER CONFINED FIRE SPACE WITHOUT FULL

BUNKER GEAR (HELMET WITH FACE SHIELD, BUNKER COATS, GLOVES AND RUBBER

BOOTS) INCLUDING A POSITIVE PRESSURE NIOSH APPROVED SELF-CONTAINED

BREATHING APPARATUS.  COOL FIRE EXPOSED CONTAINERS WITH WATER.

SECTION 6 – ACCIDENTAL RELEASE MEASURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED: COMBUSTIBLE!

ELIMINATE ALL IGNITION SOURCES.  WEAR APPROPRIATE RESPIRATOR AND PROTECTIVE CLOTHING.

WASTE DISPOSAL METHOD: IN CASE OF SPILLAGE, ABSORB WITH INERT MATERIAL AND DISPOSE OF IN ACCORDANCE WITH APPLICABLE REGULATION

SECTION 7 –  HANDLING AND STORAGE

HANDLING AND STORAGE: KEEP container closed, upright when not in use.  Store in cool, dry, well-ventilated area.  Avoid prolonged storage temperatures above 100° F.  Use caution when pouring.  Avoid breathing sanding dust.  Do not weld or flame cut on empty container.

TC7B

SECTION 8 – EXPOSURE CONTROL/PERSONAL PROTECTION

RESPIRATORY PROTECTION: WEAR AN APPROPRIATE PROPERLY FITTED RESPIRATOR (NIOSH/OSHA APPROVED) DURING AND AFTER APPLICATION.  In confined or restricted ventilation areas use AIRLINE RESPIRATORS or hoods.  Refer to appropriate regulations regarding respiratory protection.

VENTILATION:  USE WITH ADEQUATE VENTILATION.  DO NOT BREATHE VAPORS OR SPRAY MIST.

PROTECTIVE GLOVES AND CLOTHING: DEPENDENT on application method, wear resistant coveralls, gloves and shoe coverings to prevent skin contact.  Wear solvent resistant with SPLASHGUARDS or face shield to PROTECT eyes from splash, spatter and/or spray mist.  Use explosion and spark-proof equipment.  Consult APPROPRIATE regulations regarding protective clothing, glasses, and gloves.  USE NEOPRENE GLOVES IF CONTACT IS LIKELY

EYE PROTECTION: USE CHEMICAL SAFETY GOGGLES WITH SPLASH SHIELDS.

HYGIENIC PRACTICES: WASH THOROUGHLY AFTER HANDLING AND BEFORE EATING, SMOKING, OR USING THE TOILET.  LAUNDER CONTAMINATED CLOTHING BEFORE USE.  DESTROY CONTAMINATED LEATHER AND ABSORBENT SHOES, WHICH CANNOT BE DECONTAMINATED TO PREVENT REUSE.  NON-STATIC CREATING CLOTHING AND CONDUCTIVE SHOES SHOULD BE WORN.

SECTION 9 – PHYSICAL AND CHEMICAL DATA

BOILING RANGE:	323 – 392 F			WT./GALLON   9.10 #/GAL  0.2

EVAPORATION RATE:

FASTER:           		SLOWER:     xx    	THAN ETHER

% VOLATILE VOLUME: 0 %

VAPOR DENSITY:

HEAVIER:     xx    	LIGHTER:           	THAN AIR

EPA MIXED VOC:  	0 LB/GAL

TC7B

SECTION 10 – STABILITY AND REACTIVITY DATA

STABILITY: THIS PRODUCT IS STABLE UNDER NORMAL STORAGE CONDITIONS.

HAZARDOUS POLYMERIZATION: WILL NOT OCCUR UNDER NORMAL STORAGE

CONDITIONS.

HAZARDOUS DECOMPOSITION PRODUCTS: CARBON MONOXIDE, CARBON DIOXIDE

CONDITIONS TO AVOID: HEAT, SPARKS, FLAME AND CONTACT WITH STRONG OXIDIZING AGENTS, STRONG ACIDS AND ALKALI.

INCOMPATIBILITY (MATERIALS TO AVOID): STRONG OXIDIZERS, ACIDS, AND ALKALIES.

SECTION 11 – TOXICOLOGICAL PROPERTIES

SEE SECTION 2

SECTION 12 – ECOLOGICAL INFORMATION

NO INFORMATION AVAILABLE

SECTION 13 – DISPOSAL CONSIDERATIONS

EPA Waste No.: NONE

DISPOSAL METHOD: PLACE in separate, appropriate, closed container in accordance with all applicable local, State, and Federal regulations.  This material has not been tested by Toxicity Characteristic Leaching Procedure (TCLP.)

TC7B

SECTION 14 – TRANSPORTATION INFORMATION

DOT PROPER SHIPPING NAME: PAINT RELATED MATERIAL

DOT HAZARD CLASS: 8			HAZARD SUBCLASS: NA

DOT UN/N NUMBER: UN3066			PACKING GROUP: III

SECTION 15 – REGULATORY INFORMATION

U.S. FEDERAL REGULATIONS:

OSHA:  Hazardous by definition of Hazard Communication Standard (29 CFR 1910.1200)

.

INTERNATIONAL REGULATIONS:

CANADIAN WHMIS: THIS MSDS has been prepared in compliance with Controlled Product Regulations except that 16 sections were used rather than the 9 sections listed.  Canadian WHMIS Class: NO information AVAILABLE.

SECTION 16 – OTHER INFORMATION  -- NONE

HMIS:	health: 2	Flammability: 1	reaCtivity: 1

THE INFORMATION CONTAINED HEREIN IS BASED ON THE DATA AVAILABLE TO US AND IS BELIEVED TO BE CORRECT.  AMERICAN COATINGS MAKES NO WARRANTY, EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THESE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.  AMERICAN COATINGS ASSUMES NO RESPONSIBILITY FOR INJURY FROM THE USE OF THE PRODUCT DESCRIBED HEREIN.